Technische Fiche Grote Zaal               English version on page 4                                (versie: 27/01/2022)

Adres:
Ganzenmarkt 14
3512 GD Utrecht

Navigatieadres:

Minrebroederstraat 1, Utrecht

(dan nog 50m rechtdoor rijden)

Contact:

Coördinator techniek: Gibson Houwer

techniek@theaterkikker.nl  
Hoofd techniek: Orit Freedman

orit.freedman@theaterkikker.nl 
Tel. techniek: 030 – 7210756

Tel.  algemeen: 030 – 2319666

Bereikbaarheid, Laden en Lossen

Theater Kikker bevind zich midden in de historische binnenstad van Utrecht, houdt daarom rekening met de onderstaande zaken. Wij doen ons best om zo veel mogelijk obstakels te ondervangen en op te lossen.

Route en parkeren:

In verband met veel éénrichtingsverkeer en een milieuzone in Utrecht centrum,

is het handig om even te kijken op deze pagina.

Voetgangersgebied, Aslast- en lengtebeperking:

Voor het binnenrijden van het voetgangersgebied buiten de venstertijden hebben wij een ontheffing.

Meer informatie vind je hier.

Voor overschrijding van de maximale aslast van 2,0t voor de deur hebben wij een ontheffing.

Voor overschrijding van de maximale lengte van 9,00m moet de chauffeur zelf een ontheffing aanvragen bij de gemeente.

Meer informatie vind je hier.

Afmetingen en inrichting

Afmetingen:
Met afstopping : 10,50m x 9,00m (bxd).
Zonder afstopping: 11,00m x 9,40m (bxd).
Met ingeschoven tribune: 13,50m / 14,00m (d). 9,40m (b) tussen trappen.

Hoogte: vrije hoogte onder zaalbruggen: 5,85 m.

Doorgang:
Deuren: laad- en losdeuren: 1,52m x 2,08m (bxh).
Kleinste doorgang naar kleedkamers: 0,83m x 2,05m (bxh).

Vloer:
Houten zwevende vloer: 11,00m x 9,00m.
Balletvloer (zwart, wit, grijs): 10,50m x 9,00m.

Afstopping:
Achter- en zijdoek: nachtblauw, middels gordijnrails.
Horizon: filled cloth, 9,50m x 7,30m (bxh).

Rigging:
3x Motortrek tussen brug 3/4, brug 4/5 en brug 5/achterwand.

Elektra:

2x 63A CEE-Form (63A afgezekerd) links en rechts achterop het speelvlak.

Alle wandcontactdozen en dimmeraansluitingen zijn voorzien van schuko-aansluitingen.

Licht

Armaturen:
51x 	ETC S4 LED series 2 - Lustr met de volgende lenshuizen:

25x 	profiel 25°/50°.

10x 	profiel 15°/30°.

12x 	PC.

4x 	Horizon.

Conventioneel licht, in overleg te gebruiken:

30x 	1kW Par 64.

4x 	2kW ADB Fresnel met barndoors.
9x 	1kW ADB Horizon armatuur.

Dimmers / vaste spanning:
Elke brug heeft twee groepen vaste spanning verdeeld over 4 wandcontactdozen.

Alle dimmers hebben een vermogen van 2kw:

44x	verdeeld over de bruggen.
6x	links op de vloer.
6x	rechts op de vloer.

7x	links in de brug middels harting 8.

7x	rechts in de brug middels harting 8.

Aansturing:

Lichtcomputer: ETC ION + Faderwing.

Indien het gezelschap eigen lichtaansturing meeneemt, is de situatie als volgt: het kleuren van de lampen wordt gedaan door onze lichttafel. De gasttechnicus werkt met een enkele dimmerkanaal per lamp (zoals bij een conventionele lamp). Op de huislijn is DMX adres 175 t/m 512 beschikbaar voor eigen licht. Ook is er een vrije tweede lijn beschikbaar.

Overig:

Hazer: Martin K1.

Rookmachine: Rosco Vapour.

Losse dimmer: TTL Dimblock (32A, 12 kanaals).

Geluid
mengtafel: Soundcraft Expression SI (digitaal).

Luidsprekers:

Main:

4x 	Martin Audio XD12 	(12″+1″ top)  	in de diepte te verplaatsen middels rails.

4x	Martin Audio SX118 	(18″ sub)	standaard opgesteld onder de tribune.

Centercluster:

2x	Martin Audio X10 	(10″+1″ top)	in de 3e en 4e brug.

Delay:

2x 	Martin Audio X8 	(8″+1″ wedge)	voor de twee “vleugels” van de tribune.

Monitors:

4x 	Martin Audio F10+ 	(10″+1″ wedge)
Versterkers / processor:
1x 	Xilica Solaro FR1 + remote middels tablet.

1x	Martin Audio iK42 	(4x 1500w)

1x	Martin Audio iK81 	(8x 1250w)
1x	Powersoft M50Q 	(4x 750w)

Microfoons:

5x 	Shure sm58
3x 	Shure sm57
1x 	Shure beta87c (cond.)
1x 	Sennheiser e602
2x 	Sennheiser e604
2x 	Neumann KM184 (cond.)
2x 	Audio Technica ATM450 (cond.)
1x 	Octava mk012 (cond.)
4x 	BSS AR-133 (actieve DI)
4x 	Radial ProD2 (passieve stereo DI)

2x	Sennheiser EW300 G3 (handheld of beltpack)

Infrastructuur:

Tribune: 		16x XLR Send, 8x XLR Return + CAT7.

Toneel Links: 		16x XLR Send, 8x XLR Return + CAT7.
Toneel Rechts: 		8x XLR Send + CAT7.

Projectie

Projector: Panasonic PT-DZ870 (8500 lumen, WUXGA/Full HD).
Scherm: Horizondoek of Stumpfl Vario64 opzichtscherm (6,00m x 3,45m).

Infrastructuur: CAT7 patch: cabine, tribune, toneel links, toneel rechts, brug 1.

Overig

Podiumdelen: 12x 	2,00 x 1,00m.

2x 	2,00 x 0,50m.

2x 	1,00 x 1,00m (driehoek).

Pootjes 20 / 40 / 60 / 80 / 100 cm.

Ballettoren:	6x met buizen op 60, 120 en 180 cm.

Piano:		Yamaha P116 upright, zwart (op aanvraag).

Technical Rider Grote Zaal                                                                              English

Adress:
Ganzenmarkt 14
3512 GD Utrecht

Navigate to:

Minrebroederstraat 1, Utrecht

(and drive straight on for about 50m)

Contact:

Technical Coördinatior: Gibson Houwer

techniek@theaterkikker.nl  
Chief Technical dept.: Orit Freedman

orit.freedman@theaterkikker.nl 
Phone Technical dept: 030 – 7210756

Phone General: 030 – 2319666

Accesibility

Theater Kikker is located in the hostorical centre of Utrecht, therefore some of the following things must be taken into consideration. We try our best to keep this document up to date and complete. Unfortunately, some of the information from the local municipality is only available in dutch.

Navigation and parking:

We do not have a spot to park cars/vans/trucks at the theater. We do have 1 parking permit per hall, to park for free in the city centre. For trucks we have a couple of options just outside the city centre.

Because of the situation with one-way streets and the low-emission zone, it might be helpful to check this page.

Pedestrian zone, axle load and vehicle restrictions:

To drive within the pedestrian zone, we have a permit. More information can be found here (in dutch)

To exceed the maximum axle load of 2,0t, we have a permit

To exceed the lenght restriction of max. 9,00m  the driver should get a permit with the local municipality.

More information can be found here (in dutch).

Stage

Black box theatre with performances on the floor and audience on a tribune.

Dimensions:
Stage size: 11,00m x 9,00m (wxd)

Between masking: 10,50m x 9,00m (wxd)

Without tribune: 13,50m / 14,00m (d). 9,40m (b) between the stairs.
Height: clearance under lighting bridges 5,85m

Access:

Doors: loadingdoors: 1,52m x 2,08m (wxh)
Smallest door to dressing room: 0,83m x 2,05m (wxh)

Floor:

Sprung wooden deck: 11,00m x 9,00m.
Marley floor (black/white/grey): 10,50m x 9,00m.

Curtains:

Masking curtains back and sides: nightblue velvet, on rails.

Cyclorama: white, filled cloth. 9,50m x 7,30m

Flying:

3x motorized flybars: in between bridge 3/4, bridge 4/5, and bridge 5/back wall.

Electrics:

2x 63Amps CEE-form left and right on the back wall.

All wallsockets and dimmer connections are fitted for schuko plugs.

Light

fixtures:
51x 	ETC S4 LED series 2 - Lustr with the following lensetubes:

25x 	profile 25°/50°.

10x 	profile 15°/30°.

12x 	PC.

4x 	Cyclorama Projector.

Conventional halogen fixtures, upon request:

30x 	1kW Par 64.

4x 	2kW ADB Fresnel with barndoors.
9x 	1kW ADB Asymmetric Cyclorama Projector.

Dimmers / 230v outlets:
Every light bridge has two groups of four 230V outlets.

All dimmers have a rating of 2kw:

44x	distributed across the bridges.
6x	stage floor left.
6x	stage floor right.

7x	left in the bridge through harting 8.

7x	right in the bridge through harting 8.

Control:

Lighting Console: ETC ION + Faderwing.

When the company brings their own console this is our procedure:

The guest technician uses the dimmer channels, just as you would with conventional fixtures, colouring is done by our console.

On DMX line “A” (house) channels 175 t/m 512 are available.

On DMX line “B” (guest) channels 1 t/m 512 are available.

Various:

Hazer: Martin K1.

Smoke mahine: Rosco Vapour.

Portable dimmer: TTL Dimblock (32A, 12 channels).

Sound
Sound desk: Soundcraft Expression SI (digital).

Speakers:

Main:

4x 	Martin Audio XD12 	(12″+1″ top)  	mobile, on rails.

4x	Martin Audio SX118 	(18″ sub)	placed under the tribune.

Centercluster:

2x	Martin Audio X10 	(10″+1″ top)	in the 3rd en 4th bridge.

Delay:

2x 	Martin Audio X8 	(8″+1″ wedge)	for the back of the tribune.

Monitors:

4x 	Martin Audio F10+ 	(10″+1″ wedge)
Amplification/ processing:
1x 	Xilica Solaro FR1 + remote via tablet.

1x	Martin Audio iK42 	(4x 1500w)

1x	Martin Audio iK81 	(8x 1250w)
1x	Powersoft M50Q 	(4x 750w)

Microphones:

5x 	Shure sm58
3x 	Shure sm57
1x 	Shure beta87c (cond.)
1x 	Sennheiser e602
2x 	Sennheiser e604
2x 	Neumann KM184 (cond.)
2x 	Audio Technica ATM450 (cond.)
1x 	Octava mk012 (cond.)
4x 	BSS AR-133 (active DI)
4x 	Radial ProD2 (passive stereo DI)

2x	Sennheiser EW300 G3 (handheld or beltpack)

Infrastructure:

Tribune: 		16x XLR Send, 8x XLR Return + CAT7.

Stage Left: 		16x XLR Send, 8x XLR Return + CAT7.
Stage Right: 		8x XLR Send + CAT7.

Projection

Projector: Panasonic PT-DZ870 (8500 lumen, WUXGA/Full HD).
Screen: Cyclorama screen or Stumpfl Vario64 screen (6,00m x 3,45m).

Infrastructure: CAT7 patch: cabine, tribune, stage left, stage right, bridge 1.

Various

Stage decks:	12x 	2,00 x 1,00m.

2x 	2,00 x 0,50m.

2x 	1,00 x 1,00m (triangle).

heights 20 / 40 / 60 / 80 / 100 cm.

Light booms:	6x with tube heights 60, 120 and 180 cm.

Piano:		Yamaha P116 upright, black (on request).